PRACTICAL NO 5: Configuring IPv6 ACLs

Access Control Lists for IPv6 Traffic Filtering

The standard ACL functionality in IPv6 is similar to standard ACLs in IPv4. Access lists determine what traffic is blocked and what traffic is forwarded at device interfaces and allow filtering based on source and destination addresses, inbound and outbound to a specific interface. Each access list has an implicit deny statement at the end. IPv6 ACLs are defined and their deny and permit conditions are set using the ipv6 access-list command with the deny and permit keywords in global configuration mode.

IPv6 extended ACLs augments standard IPv6 ACL functionality to support traffic filtering based on IPv6 option headers and optional, upper-layer protocol type information for finer granularity of control (functionality similar to extended ACLs in IPv4).

IPv6 Packet Inspection

Access Class Filtering in IPv6

IPv6 Packet Inspection

The following header fields are used for IPv6 inspection: traffic class, flow label, payload length, next header, hop limit, and source or destination IP address. For further information on and descriptions of the IPv6 header fields, see RFC 2474.

Access Class Filtering in IPv6

Filtering incoming and outgoing connections to and from the device based on an IPv6 ACL is performed using the ipv6 access-class command in line configuration mode. The ipv6 access-class command is similar to the access-class command, except the IPv6 ACLs are defined by a name. If the IPv6 ACL is applied to inbound traffic, the source address in the ACL is matched against the incoming connection source address and the destination address in the ACL is matched against the local device address on the interface. If the IPv6 ACL is applied to outbound traffic, the source address in the ACL is matched against the local device address on the interface and the destination address in the ACL is matched against the outgoing connection source address. We recommend that identical restrictions are set on all the virtual terminal lines because a user can attempt to connect to any of them.

We use the following topology

Configuring PC1

Configuring PC0

Configuring Server0

For setting the ipv6 addresses we need to use the CLI mode for each Router as follows

Configuring Router0

Router>

Router>enable

Router#

Router#configure terminal

Router(config)#ipv6 unicast-routing

Router(config)#interface GigabitEthernet0/0

Router(config-if)#ipv6 address 2002::1/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#interface GigabitEthernet0/1

Router(config-if)#ipv6 address 2001::1/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#interface Serial0/1/0

Router(config-if)#ipv6 address 2003::1/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Configuring Router1

Router>enable

Router#configure terminal

Router(config)#ipv6 unicast-routing

Router(config)#

Router(config)#interface Serial0/1/0

Router(config-if)#ipv6 address 2003::1/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#

Router(config-if)#exit

Router(config)#

Router(config)#interface Serial0/1/1

Router(config-if)#ipv6 address 2004::1/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Configuring Router2

Router>enable

Router#configure terminal

Router(config)#ipv6 unicast-routing

Router(config)#

Router(config)#interface Serial0/1/1

Router(config-if)#ipv6 address 2004::2/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface GigabitEthernet0/0

Router(config-if)#ipv6 address 2005::1/64

Router(config-if)#ipv6 rip a enable

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Check the connectivity by pinging from PCs to Server

And we see that the connectivity is established

We configure the ACL and apply it to the Router1 with the following conditions

- No HTTP or HTTPS allowed on server by any host

- No www service accessible on the server by any host

- Only ipv6 packets allowed towards the server

We enter the following commands in the CLI mode of the  Router1 and apply it at the proper interface

Router>

Router>enable

Router#configure terminal

Router(config)#ipv6 access-list smile

Router(config-ipv6-acl)#deny tcp any host 2005::2 eq www

Router(config-ipv6-acl)#deny tcp any host 2005::2 eq 443

Router(config-ipv6-acl)#permit ipv6 any any

Router(config-ipv6-acl)#

Router(config-ipv6-acl)#exit

Router(config)#

Router(config)#interface Serial0/1/1

Router(config-if)#ipv6 traffic-filter smile in

Router(config-if)#exit

Router(config)#

We verify the configuration by first accessing the www service from the browser of both PCs and get failure

Next we verify whether the ipv6 protocol works by pinging server from any of the PC (it must be successful)

Hence the given ACLs have been applied and verified on host running on ipv6 protocol